8088/286/386/486 Instruction Set

1) Instruction Set Matrix

2) Secondary Opcode Space

3) Arithmetic

4) Data Transfer

5) Execution Control

6) Processor Control

7) Protection Control  (80286)

8) String Operation

9) 386/486-specific Opcodes (80386/80486)

10) 80x87 Floating Point Opcodes (8087,287,387,486)

------------------------------------------------------------------------------

1)Instruction Set Matrix

========================

indicates 286 only (not available in 8088/8086).

indicates  386  and/or  486

first row, then col

╔════════Š════════Š════════Š════════Š════════Š════════Š════════Š════════©

║   x0   │   x1   │   x2   │   x3   │   x4   │   x5   │   x6   │   x7   │

═══╬════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Į

0x ║ ADD  + │ ADD  + │ ADD  + │ ADD  + │ ADD  + │ ADD  + │ PUSH + │ POP +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ ES     │ ES     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

1x ║ ADC  + │ ADC  + │ ADC  + │ ADC  + │ ADC  + │ ADC  + │ PUSH + │ POP +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ SS     │ SS     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

2x ║ AND  + │ AND  + │ AND  + │ AND  + │ AND  + │ AND  + │ SEG  + │ DAA +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ ES:    │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

3x ║ XOR  + │ XOR  + │ XOR  + │ XOR  + │ XOR  + │ XOR  + │ SEG  + │ AAA +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ SS:    │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

4x ║ INC  + │ INC  + │ INC  + │ INC  + │ INC  + │ INC  + │ INC  + │ INC +  │

║ AX     │ CX     │ DX     │ BX     │ SP     │ BP     │ SI     │ DI     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

5x ║ PUSH + │ PUSH + │ PUSH + │ PUSH + │ PUSH + │PUSH  + │ PUSH + │ PUSH + │

║ AX     │ CX     │ DX     │ BX     │ SP     │BP      │ SI     │ DI     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

6x ║ PUSHA +│ POPA + │ BOUND +│ ARPL+  │ SEG FS+│ SEG GS+│ opSize │ addrSiz│

║        │        │        │        │        │        │ prefix │ prefix │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

7x ║ JO   + │ JNO  + │ JB/  + │ JNB/ + │ JE/  + │ JNE/ + │ JBE/ + │ JNBE/ +│

║        │        │ JNAE   │ JAE    │ JZ     │ JNZ    │ JNA    │ JA     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

8x ║ ArOp1 +│ ArOp1 +│ ArOp2 +│ ArOp2+ | TEST + │ TEST + │ XCHG + │ XCHG  +│

║r/m,im8 │r/m,im16│r/m8,im8│rm16,im8│r/m,r8  │r/m,r16 │r8,r/m  │r16,r/m │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

9x ║ NOP  + │ XCHG + │ XCHG + │ XCHG + │ XCHG + │ XCHG + │ XCHG + │ XCHG  +│

║        │ AX,CX  │ AX,DX  │ AX,BX  │ AX,SP  │ AX,BP  │ AX,SI  │ AX,DI  │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Ax ║ MOV    │ MOV    │ MOV    │ MOV    │ MOVSB +│ MOVSW+ │ CMPSB+ │ CMPSW +│

║AL,mem8 │AX,mem16│mem8,AL │mem16,AX│        │        │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Bx ║ MOV  + │ MOV  + │ MOV  + │ MOV  + │ MOV  + │ MOV +  │ MOV  + │ MOV +  │

║ AL,im8 │ CL,im8 │ DL,im8 │ BL,im8 │ AH,im8 │ CH,im8 │ DH,im8 │ BH,im8 │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Cx ║ ShfOp+ │ ShfOp +│RET near│RET near│ LES  + │ LDS +  │ MOV  + │ MOV +  │

║r/m8,imm│r/m16,im│ ±im16+ │   +    │r16,mem │r16,mem │mem,im8 │mem,im16│

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Dx ║ ShfOp  │ ShfOp  │ ShfOp  │ ShfOp  │ AAM  + │ AAD  + │      + │ XLAT + │

║r/m8,1  │r/m16,1 │r/m8,CL │r/m16,CL│        │        │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Ex ║ LOOPNE/│ LOOPE/ │ LOOP   │ JCXZ   │ IN     │ IN     │ OUT    │ OUT    │

║ LOOPNZ │ LOOPZ  │        │ JECXZ  │AL,port8│AX,port8│AL,port8│AX,port8│

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Fx ║ LOCK   │        │ REP/ + │ REPZ/+ │ HALT + │ CMC  + │ Grp1   │ Grp1   │

║        │    +   │ REPNE  │ REPE   │        │        │r/m8    │r/m16   │

═══╬════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Į

║   x0   │   x1   │   x2   │   x3   │   x4   │   x5   │   x6   │   x7   │

╚════════¤════════¤════════¤════════¤════════¤════════¤════════¤════════

╔════════Š════════Š════════Š════════Š════════Š════════Š════════Š════════©

║   x8   │   x9   │   xA   │   xB   │   xC   │   xD   │   xE   │   xF   │

═══╬════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Į

0x ║ OR   + │ OR   + │ OR +   │ OR  +  │ OR  +  │ OR  +  │ PUSH + │ Extnsn │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ CS     │ OpCode │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

1x ║ SBB  + │ SBB  + │ SBB  + │ SBB +  │ SBB +  │ SBB +  │ PUSH + │ POP  + │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ DS     │ DS     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

2x ║ SUB +  │ SUB +  │ SUB +  │ SUB  + │ SUB +  │ SUB +  │ SEG  + │ DAS +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ CS     │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

3x ║ CMP +  │ CMP +  │ CMP +  │ CMP +  │ CMP +  │ CMP +  │ SEG  + │ AAS +  │

║ r/m,r8 │r/m,r16 │ r8,r/m │r16,r/m │ AL,im8 │AX,im16 │ DS     │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

4x ║ DEC +  │ DEC +  │ DEC +  │ DEC +  │ DEC +  │ DEC +  │ DEC +  │ DEC +  │

║ AX     │ CX     │ DX     │ BX     │ SP     │ BP     │ SI     │ DI     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

5x ║ POP +  │ POP +  │ POP +  │ POP +  │ POP +  │POP  +  │ POP +  │ POP +  │

║ AX     │ CX     │ DX     │ BX     │ SP     │BP      │ SI     │ DI     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

6x ║ PUSH + │  IMUL+ │  PUSH +│  IMUL +│  INSB +│  INSW+ │ OUTSB+ │ OUTSW+ │

║ imm16  │r/m,im16│ imm8   │r/m,im8 │        │        │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

7x ║ JS  +  │ JNS  + │ JP/ +  │ JNP/ + │ JL/  + │ JNL/ + │ JLE/ + │ JNLE/ +│

║        │        │ JPE    │ JPO    │ JNG    │ JGE    │ JNG    │ JG     │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

8x ║ MOV  + │ MOV  + │ MOV +  │ MOV +  │ MOV  + │ LEA +  │ MOV +  │ POP  + │

║r/m,r8  │r/m,r16 │r8,r/m  │r16,r/m │r/m,seg │r16,mem │seg,r/m │ r/m    │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

9x ║ CBW +  │ CWD  + │ CALL   │ WAIT + │ PUSHF+ │ POPF+  │ SAHF + │ LAHF + │

║        │        │ far    │        │        │        │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Ax ║ TEST + │ TEST + │ STOSB +│ STOSW +│ LODSB+ │ LODSW+ │ SCASB +│ SCASW+ │

║AL,mem8 │AX,mem16│        │        │        │        │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Bx ║ MOV +  │ MOV  + │ MOV +  │ MOV +  │ MOV +  │ MOV +  │ MOV +  │ MOV +  │

║AX,im16 │CX,im16 │DX,im16 │BX,im16 │SP,im16 │BP,im16 │SI,im16 │DI,im16 │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Cx ║ ENTER +│ LEAVE +│RET far+│RET far+│ INT 3+ │ INT +  │ INTO + │ IRET + │

║im16,im8│        │ ±im16  │        │        │ im8    │        │        │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Dx ║ ESC 0  │ ESC 1  │ ESC 2  │ ESC 3  │ ESC 4  │ ESC 5  │ ESC 6  │ ESC 7  │

║ 387/486│ 387/486│ 387/486│ 387/486│ 387/486│ 387/486│ 387/486│ 387/486│

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Ex ║ CALL + │ JMP +  │ JMP +  │ JMP  + │ IN  +  │ IN  +  │ OUT  + │ OUT +  │

║ near   │ near   │ far    │ short  │ AL,DX  │ AX,DX  │ AL,DX  │ AX,DX  │

───Ī────────┼────────┼────────┼────────┼────────┼────────┼────────┼────────┤

Fx ║ CLC  + │ STC +  │ CLI +  │ STI  + │ CLD +  │ STD  + │ Grp2   │ Grp3   │

║        │        │        │        │        │        │ r/m8   │ r/m16  │

═══╬════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Ļ════════Į

║   x8   │   x9   │   xA   │   xB   │   xC   │   xD   │   xE   │   xF   │

╚════════¤════════¤════════¤════════¤════════¤════════¤════════¤════════

1)Instruction Set

-----------------------------------------------------------------------------

2) Secondary Opcode Space

=========================

This table identifies operations for the instruction groups which contain the

opcode in bits 3-5 of the second byte (normally the addressing mode byte).

╔═══════Š═══════Š═══════Š═══════Š═══════Š═══════Š═══════Š═══════©

║md000rm│md001rm│md010rm│md011rm│md100rm│md101rm│md110rm│md111rm│

═══════╬═══════Ļ═══════Ļ═══════Ļ═══════Ļ═══════Ļ═══════Ļ═══════Ļ═══════Į

ArOp1  ║ ADD   │ OR    │ ADC   │ SBB   │ AND   │ SUB   │ XOR   │ CMP   │

───────Ī───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

ArOp2  ║ ADD   │       │ ADC   │ SBB   │       │ SUB   │       │ CMP   │

───────Ī───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

ShftOp ║ ROL   │ ROR   │ RCL   │ RCR   │SHL/SAL│ SHR   │       │ RAR   │

───────Ī───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

Grp1   ║ TEST  │       │ NOT   │ NEG   │ MUL   │ IMUL  │ DIV   │ IDIV  │

───────Ī───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

Grp2   ║ INC   │  DEC  │CALL nr│CALL fr│JMP nr │JMP far│ PUSH  │       │

───────Ī───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

Grp3   ║ INC   │  DEC  │       │       │       │       │       │       │

═══════╩═══════¤═══════¤═══════¤═══════¤═══════¤═══════¤═══════¤═══════

2)Instruction Set Matrix   Instruction Set

---------------------------------------------------------------------------

3) Arithmetic Instructions

==========================

ADD  dest,src         add two operands, placing result in dest

dest = (src + dest)

ADC  dest,src         add two operands, using carry from previous ADD

dest = (src + dest + CF)

INC  dest             add 1 to dest (reg or r/m)

dest = (dest+1)

SUB  dest,src         subtract src from dest, leaving result in dest

dest = (dest - src)

SBB  dest,src         subtract using borrow (carry flag) from previous SUB

dest = ((dest - src) - CF)

DEC  dest             subtract 1 from dest (reg or r/m)

dest = (dest - 1)

CMP  dest,src         compare (non-destructive subtract)

flags are set to indicate carry, overflow, etc.

NEG  dest             change sign of an operand

dest = (0 - dest);

AAA                   adjust after ASCII addition

AL = (AL corrected for ASCII addition)

AAS                   adjust for ASCII subtraction

AL = (AL corrected for ASCII subtraction)

AAM                   adjust after ASCII multiply

AH:AL = (AH:AL corrected for ASCII multiplication)

AAD                   adjust before ASCII division

AH:AL = (AH:AL prepared for division of ASCII codes)

DAA                   adjust after BCD addition

AL = (AL corrected for BCD addition)

DAS                   adjust for BCD subtraction

AL = (AL corrected for BCD subtraction)

MUL src               multiply AL(AX) by unsigned value src (reg/mem)

AX = (AL * src8)

DX:AX = (AX * src16)

IMUL src              multiply AL(AX) by signed integer src (reg/mem)

AX = (AL * immed8)

DX:AX = (AX * immed16)

IMUL reg16,r/m,im16   multiply reg/mem by 16-bit immediate signed value ¦286Ģ

reg16 = (r/m8 * im16)

reg16 = (r/m16 * im16)

DIV  src              divide accumulator by unsigned value src (reg/mem)

Division by 0 or result too large causes INT 0.

AL = (AX ÷ src8);     AH = (AX MOD src8)

AX = (DX:AX ÷ src16); DX = (DX:AX MOD src16)

IDIV src              divide accumulator by unsigned value src (reg/mem)

CBW                   convert byte to word (sign-extend AL into AH)

AH = (filled with bit-7 of AL)

CWD                   convert word to dword (sign-extend AX into DX)

DX = (filled with bit-15 of AX)

AND dest,src          logical AND (mask; reset dest bits which are 0s in src)

dest = (dest & src)

TEST dest,src         non-destructive AND (bit test; JZ will jump if no match)

flags = (set as for dest & src)

OR   dest,src         inclusive OR (set dest bits which are 1s in src)

dest = (dest | src)

XOR  dest,src         exclusive OR (toggle dest bits which are 1s in src)

dest = (dest ^ src)

NOT  dest             form 1's complement (toggle every bit in dest)

dest = (~dest)

──────────────────────────────────────────────────────────────────────────────

3) Instruction Set

4) Data Transfer Instructions

=============================

MOV  dest,src        transfer (copy) data to/from register, to/from memory

dest = src

XCHG dest,src        exchange values of two registers or register and r/m

dest <-> src

IN port8 (or DX)     input to AL(or AX) from I/O port

byte: AL = port

word: AL = [port]; AH = [port+1]

OUT port8 (or DX)    output from AL(or AX) to I/O port

byte: [port] = AL

word: [port] = AL [port+1] = AH

XLAT                 translate AL into a value in a translation table at BX

AL = ES:[BX+(AL)]

LEA  reg16,addr      load address into a register

reg16 = (result of EA calculation of addr offset)

LDS  reg16,mem       load DS and reg16 from memory variable

reg16 = [mem16]; DS = [mem16+2]

LES  reg16,mem       load ES and reg16 from memory variable

reg16 = [mem16]; ES = [mem16+2]

LAHF                 load/convert flags into AH

SAHF                 store/convert AH into flags

PUSH src             transfer reg16 or r/m16 to stack

SP-=2; SS:[SP] = src

PUSH immed           transfer immed16 (or sign-extended immed8) to stack.

SP-=2; SS:[SP] = immed16    ¦286Ģ only.

PUSHA                PUSH ALL: copy general registers to stack.  ¦286Ģ only.

SP-=10H; AX,BX,CX,DX,SI,DI,BP,SP saved on stack

PUSHF                PUSH Flags: transfer flags register to stack

SP-=2; SS:[SP] = flags

POP dest             transfer from stack to dest (reg16, r/m16, or segreg)

dest = SS:[SP]; SP+=2

POPA                 POP All: copy general registers from stack.  ¦286Ģ only.

SP+=10H; AX,BX,CX,DX,SI,DI,BP restored from stack

POPF                 POP Flags: transfer from stack to flags register

flags = SS:[SP]; SP+=2

──────────────────────────────────────────────────────────────────────────────

4)String Operation Instructions                                Instruction Set

5) Execution Control Instructions

=================================

JMP     target       unconditionally transfer control to target

short: IP=(IP+(target displacement sign-extended))

near: IP=(IP+(target displacement))

indirect: IP=(register or value in memory)

far: SS=target_seg; IP target_offset

JCXZ    short_label  jump if CX==0

LOOP    short_label  CX (CX-1) jump if CX!=0

LOOPE/  short_label  CX (CX-1) jump if CX!=0 && ZF==ZR==1

LOOPZ

LOOPNE/ short_label  CX (CX-1) jump if CX!=0 && ZF==NZ==0

LOOPNZ

Jccc    short_label  jump when condition ccc is met

IP=(IP+(8-bit displacement sign-extended to 16 bits))

JA/JNBE short_label  jump if Above ((CF & ZF)==0 after unsigned math)

JAE/JNB short_label  jump if Above or Equal (CF==NC==0 after unsigned math)

JB/JC   short_label  jump if Below/Jump if Carry set (CF==CY==1)

JE/JZ   short_label  jump if Equal (ZF==ZR==1)

JG/JNGE short_label  jump if Greater (SF==(OF & ZF) after signed math)

JGE/JNL short_label  jump if Greater or Equal (SF==OF after signed math)

JL/JNGE short_label  jump if Less (ZF != OF after signed math)

JLE/JNG short_label  jump if Less or Equal (SF!=OF || ZF==0 after signed math)

JNC     short_label  jump if carry not set (CF==NC==0) (same as JAE/JNB)

JNE/JNZ short_label  jump if Not Equal (ZF==NZ==0)

JNO     short_label  jump if Not Overflow (OF==NO==0)

JNP/JPO short_label  jump if Parity Odd (PF==PO==0: count of 1-bits is ODD)

JNS     short_label  jump if Not Sign (SF==PL==0: same as high-bit of dest)

JO      short_label  jump if Overflow (OF==OV==1)

JP/JPE  short_label  jump if Parity Even (PF==PE==1 count of 1-bits is EVEN)

JS      short_label  jump if Sign (SF==NG==1: same as high-bit of dest)

BOUND reg16,lmts     perform limit-check on reg16.  lmts is the address of a

2-word table with desired min/max limits.  ¦286Ģ only.

if (reg16<DS:[lmts]) or (reg16>DS:[lmts+2]) then INT 5

ENTER frmsiz,frms    set high-level language stack frame.  Use as the first

operation in a CALLed procedure.  ¦286Ģ only.  Same as:

PUSH BP;      (repeated frms times)

MOV BP,SP;

PUSH SP;

SUB SP,frmsiz; (allocate dynamic space on stack)

LEAVE                undo the effect of ENTER.  Use just before RET.

Restores SP and BP to values at time of ENTER.

INT type             perform a software interrupt (call a system function)

PUSHF; IF 0; TF 0;

PUSH CS; PUSH IP

IP = 0000:[type * 4]; CS = 0000:[(type * 4) + 2]

INTO type            if OF==OV==1, then perform INT type

IRET                 return from interrupt.  Effectively the same as:

POP IP; POP CS; POPF

──────────────────────────────────────────────────────────────────────────────

5) Instruction Set

6) Processor Control Instructions

=================================

CLC         clear the carry flag to NC                             (CF=0)

CMC         complement (reverse the value of) the carry flag       (CF=~CF)

STC         set the carry flag to CY                               (CF=1)

CLD         clear direction flag to UP (string ops auto-increment) (DF=0)

STD         set direction flag to DN (string ops auto-decrement)   (DF=1)

CLI         disable maskable hardware interrupts                   (IF=0)

STI         enable maskable hardware interrupts                    (IF=1)

CTS/CLTS    clear task switch flag.  ¦286Ģ                         (TF=1)

HLT         halt processing (perform NOPs until an interrupt occurs)

WAIT/FWAIT  wait for TEST line active (synchronize with coprocessor)

LOCK        (prefix) prevent coprocessor bus access for the next instruction

386    486  prevents access only to mem affected by next opcode

SEG segreg  (prefix) override default segreg for next EA calculation

──────────────────────────────────────────────────────────────────────────────

6) Instruction Set

7) Protection Control Instructions

==================================

All instructions on this page require ¦286Ģ or later CPU

LGDT src       load Global Descriptor Table from 6-byte table at src

SGDT dest      store 6-byte Global Descriptor Table to memory at dest

LIDT src       load Interrupt Descriptor Table from 6-byte table at src

SIDT dest      store 6-byte Interrupt Descriptor Table to memory at dest

LLDT src       load Local Descriptor Table (GDT selector) from reg/mem16

SLDT dest      store Local Descriptor Table register into dest (r/m16)

LMSW src       load Machine Status Word (use to enter protected mode)

SMSW src       store Machine Status Word to reg/mem16

LTR  src       load Task Register (GDT selector) from reg/mem16

STR  dest      store Task Register to reg/mem16

LAR  dest,src  load high-byte of dest with Access Rights of src descriptor

LSL  dest,src  load dest with Segment Limit of descriptor named by src

ARPL lvl       Adjust Requested Privilege Level to higher of current or lvl

VERR seg       Sets ZF to ZR if task has read privileges for segment seg

VERW seg       Sets ZF to ZR if task has write privileges for segment seg

──────────────────────────────────────────────────────────────────────────────

7) Instruction Set

8) String Operation Instructions

================================

CLD                  clear direction flag to UP (DF=UP=0)

sets Ł (Delta) to positive.  String ops auto-increment

STD                  set direction flag to DN (DF=DN=1)

sets Ł (Delta) to negative.  String ops auto-decrement

REP/REPE/REPZ        (prefix) repeat: perform string operation repeatedly

CX=(CX-1); string op repeats until CX==0

REPNE/REPNZ          (prefix) repeat: useful for string ops CMPS and SCAS

ZF=0; CX=(CX-1); StrOp repeats while (CX!=0 and ZF==0)

MOVSB                copy byte(word) string (byte:Ł=1, word:Ł=2)

MOVSW                 ES:[DI]=DS:[SI]; DI+=Ł; SI+=Ł

MOVSXB               copy byte(word) with SF (sign) extension  386

MOVSXW               copy byte(word) with ZF (zero) extension  386

LODSB                copy string byte(word) into AL(AX)

AL = DS:[SI]; SI+=1;

LODSW                 AX = DS:[SI]; SI+=2;

STOSB                store bytes(words) into string

ES:[DI]=AL; DI+=1;

STOSW                 ES:[DI]=AX; DI+=2;

CMPSB                compare byte(word) strings (byte:Ł=1, word:Ł=2)

CMPSW                 flags (result of CMP DS:[SI],ES:[DI]); DI+=Ł; SI+=Ł

SCASB                find byte(word) in string

flags (result of CMP DS:[DI],AL); DI+=1

SCASW                 flags (result of CMP DS:[DI],AX); DI+=2

INSB                 port input byte(word) into string (byte:Ł=1, word:Ł=2)

INSW                 ¦286Ģ only.

ES:[DI] (byte/word at port DX); DI+=Ł;

OUTSB                port output byte(word) from string (byte:Ł=1, word:Ł=2)

OUTSW                ¦286Ģ only.

[port DX] DS:[SI]; SI+=Ł;

──────────────────────────────────────────────────────────────────────────────

Instruction Set

9) 386/486-specific Opcodes

===========================

Opcodes on this page are supported by  386 . Opcodes for  486  are flagged.

BSF reg,src         Bit Scan Forward. Put index of next 1-bit of src into reg

BSR reg,src         Bit Scan Reverse. Put index of prev 1-bit of src into reg

(both: ZF is set and reg 0 if src contains all 0s)

BSWAP reg32         Swap low-to-high ordering of 4-byte reg32 to high-to-low

ordering.   486  only

BT r/m,im8          Test bit.  Put bit im8 (or reg) of reg/mem into CF.

r/m,reg           (im8 or reg is used modulo 16 or modulo 32)

BTC r/m,im8         Test bit im8 (or reg) of r/m & complement that bit.

r/m,reg

BTR r/m,im8         Test bit im8 (or reg) of reg/mem and reset that bit.

r/m,reg

BTS r/m,im8         Test bit im8 (or reg) of reg/mem and set that bit.

r/m,reg

CMPXCHG r/m,reg     Compare and Exchange.   486  only

INVD                Invalidate Data Cache.   486  only

INVLPG mem          Invalidate Translation Lookaside Buffer (TLB) entry

486  only

JECXZ target        Jump to target if ECX is 0.

LFS reg,mem         Load FS and reg from memory.

LGS reg,mem         Load GS and reg from memory.

LSS reg,mem         Load SS and reg from memory.

MOV CRn,src32       Load src32 data into Control Register n (0,2,or 3)

MOV dest,CRn        Load Control Register n into dest (reg/mem32)

MOV DRn,src32       Load src32 data into Debug Register n (0-3,6,or 7)

MOV dest,DRn        Load Debug Register n into dest (reg/mem32)

MOV TRn,src32       Load src32 data into Test Register n (3-7)

MOV dest,TRn        Load Test Register n into dest (reg/mem32)

SETccc dest         If condition ccc is true, set the byte at dest to 1;

else, set it to 0.  (ccc is the same as options of

conditional jump ops. See Execution Control Instructions)

SHLD r/m,reg,im8    Shift left r/m (16- or 32-bit) by im8 (or CL) bits,

SHLD r/m,reg,CL       inserting data from reg into the vacated positions

SHRD r/m,reg,im8    Shift right r/m (16- or 32-bit) by im8 (or CL) bits,

SHRD r/m,reg,CL       inserting data from reg into the vacated positions

WBINVD              Write-Back and Invalidate Data Cache.   486  only

XADD r/m,reg        Exchange and Add.   486  only

──────────────────────────────────────────────────────────────────────────────

Instruction Set

10) 80x87 Floating Point Opcodes

================================

These instructions can be executed when a 8087/80287/80387 coprocessor is

available or when using an 80486 CPU.

┌─────────────────────────────┐

│ Data Transfer and Constants │

└─────────────────────────────┘

FLD src                 Load real: st(0) = src (mem32/mem64/mem80)

FILD src             Load integer: st(0) = src (mem16/mem32/mem64)

FBLD src                 Load BCD: st(0) = src (mem80)

FLDZ                    Load zero: st(0) = 0.0

FLD1                       Load 1: st(0) = 1.0

FLDPI                     Load pi: st(0) = Ņ (ie, pi)

FLDL2T              Load log2(10): st(0) = log2(10)

FLDL2E               Load log2(e): st(0) = log2(e)

FLDLG2              Load log10(2): st(0) = log10(2)

FLDLN2               Load loge(2): st(0) = loge(2)

FST dest               Store real: dest = st(0) (mem32/mem64)

FSTP dest                          dest = st(0) (mem32/mem64/mem80); pop stack

FIST dest           Store integer: dest = st(0) (mem32/mem64)

FISTP dest                         dest = st(0) (mem16/mem32/mem64); pop stack

FBST dest               Store BCD: dest = st(0) (mem80)

FBSTP dest                         dest = st(0) (mem80); pop stack

┌─────────┐

│ Compare │

└─────────┘

FCOM                 Compare real: Set flags as for st(0) - st(1)

FCOM op                            Set flags as for st(0) - op (mem32/mem64)

FCOMP op                           Compare st(0) to op (reg/mem); pop stack

FCOMPP                             Compare st(0) to st(1); pop stack twice

FICOM op          Compare integer: Set flags as for st(0) - op (mem16/mem32)

FICOMP op                          Compare st(0) to op (mem16/mem32); pop

stack

FTST                Test for zero: Compare st(0) to 0.0

FUCOM st(i)     Unordered Compare: st(0) to st(i)                      486

FUCOMP st(i)                       Compare st(0) to st(i) and pop stack

FUCOMPP st(i)                      Compare st(0) to st(i) and pop stack twice

FXAM                      Examine: Eyeball st(0) (set condition codes)

┌────────────┐

│ Arithmetic │

└────────────┘

FADD                     Add real: st(0) = st(0) + st(1)

FADD src                           st(0) = st(0) + src (mem32/mem64)

FADD st(i),st                      st(i) = st(i) + st(0)

FADDP st(i),st                     st(i) = st(i) + st(0); pop stack

FIADD src             Add integer: st(0) = st(0) + src (mem16/mem32)

FSUB                Subtract real: st(0) = st(0) - st(1)

FSUB src                           st(0) = st(0) - src (reg/mem)

FSUB st(i),st                      st(i) = st(i) - st(0)

FSUBP st(i),st                     st(i) = st(i) - st(0); pop stack

FSUBR st(i),st  Subtract Reversed: st(0) = st(i) - st(0)

FSUBRP st(i),st                    st(0) = st(i) - st(0); pop stack

FISUB src        Subtract integer: st(0) = st(0) - src (mem16/mem32)

FISUBR src     Subtract Rvrsd int: st(0) = src - st(0) (mem16/mem32)

FMUL                Multiply real: st(0) = st(0) * st(1)

FMUL st(i)                         st(0) = st(0) * st(i)

FMUL st(i),st                      st(i) = st(0) * st(i)

FMULP st(i),st                     st(i) = st(0) * st(i); pop stack

FIMUL src        Multiply integer: st(0) = st(0) * src (mem16/mem32)

FDIV                  Divide real: st(0) = st(0) ÷ st(1)

FDIV st(i)                         st(0) = st(0) ÷ t(i)

FDIV st(i),st                      st(i) = st(0) ÷ st(i)

FDIVP st(i),st                     st(i) = st(0) ÷ st(i); pop stack

FIDIV src          Divide integer: st(0) = st(0) ÷ src (mem16/mem32)

FDIVR st(i),st  Divide Rvrsd real: st(0) = st(i) ÷ st(0)

FDIVRP st(i),st                    st(0) = st(i) ÷ st(0); pop stack

FIDIVR src       Divide Rvrsd int: st(0) = sr    ÷ st(0) (mem16/mem32)

FSQRT                 Square Root: st(0) = ¹ st(0)

FSCALE        Scale by power of 2: st(0) = 2^st(0)

FXTRACT          Extract exponent: st(0) = exponent of st(0); and gets pushed

st(0) = significand of st(0)

FPREM           Partial remainder: st(0) = st(0) MOD st(1)

FPREM1   Partial Remainder (IEEE): same as FPREM, but in IEEE standard  486

FRNDINT      Round to nearest int: st(0) = INT( st(0) ); depends on RC flag

FABS           Get absolute value: st(0) = ABS( st(0) ); removes sign

FCHS                  Change sign: st(0) = -st(0)

┌────────────────┐

│ Transcendental │

└────────────────┘

FCOS                       Cosine: st(0) = COS( st(0) )

FPTAN             Partial tangent: st(0) = TAN( st(0) )

FPATAN         Partial Arctangent: st(0) = ATAN( st(0) )

FSIN                         Sine: st(0) = SIN( st(0) )

FSINCOS           Sine and Cosine: st(0) = SIN( st(0) ) and is pushed to st(1)

st(0) = COS( st(0)

F2XM1         Calculate (2 ^ x)-1: st(0) = (2^st(0)) - 1

FYL2X       Calculate Y * log2(X): st(0) is Y; st(1) is X; this replaces st(0)

and st(1) with: st(0) * log2( st(1) )

FYL2XP1   Calculate Y * log2(X+1): st(0) is Y; st(1) is X; this replaces st(0)

and st(1) with: st(0) * log2( st(1)+1 )

┌───────────────────┐

│ Processor Control │

└───────────────────┘

FINIT              Initilaize FPU

FSTSW AX        store Status word: AX = MSW

FSTSW dest                         dest = MSW (mem16)

FLDCW src       Load control word: FPU CW = src (mem16)

FSTCW dest     Store control word: dest   = FPU CW

FCLEX            Clear exceptions

FSTENV dest     Store environment: store status, control and tag words and

exception pointers into memory at dest

FLDENV src       Load environment: load environment from memory at src

FSAVE dest        Store FPU state: store FPU state into 94-bytes at dest

FRSTOR src         Load FPU state: restore FPU state as saved by FSAVE

FINCSTP   Increment FPU stack ptr: st(6) st(5); st(5) st(4),...,st(0) ?

FDECSTP   Decrement FPU stack ptr: st(0) st(1); st(1) st(2),...,st(7) ?

FFREE st(i)    Mark reg st(i) as unused

FNOP                 No operation: st(0) = st(0)

WAIT/FWAIT  Synchronize FPU & CPU: Halt CPU until FPU finishes current opcode.

──────────────────────────────────────────────────────────────────────────────

Flags

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00

┌──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┬──┐

│  │  │  │  │OF│DF│IF│TF│SF│ZF│  │AF│  │PF│  │CF│

└──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┴──┘

│  │  │  │  │   │     │     │     │

│  │  │  │  │   │     │     │     └─ Carry     Flag

│  │  │  │  │   │     │     └─────── Parity    Flag

│  │  │  │  │   │     └───────────── Auxiliary Flag

│  │  │  │  │   └─────────────────── Zero      Flag

│  │  │  │  └─────────────────────── Sign      Flag

│  │  │  └────────────────────────── Trap      Flag

│  │  └───────────────────────────── Interrupt-Enable Flag

│  └──────────────────────────────── Direction Flag

└─────────────────────────────────── Overflow  Flag